Domanda 1

Considerando il processore MIPS64 e l’architettura descritta in seguito:

| - Integer ALU: 1 clock cycle - Data memory: 1 clock cycle - FP multiplier unit: pipelined 6 stages | - FP divider unit: not pipelined unit that requires 7 clock cycles - FP arithmetic unit: pipelined 4 stages - branch delay slot: 1 clock cycle, and the branch delay slot disabled | - forwarding enabled - it is possible to complete instruction EXE stage in an out-of-order fashion. |
| --- | --- | --- |

Usando il frammento di codice riportato, si calcoli il tempo di esecuzione dell’intero programma in colpi di clock e si completi la seguente tabella.

;	for (i = 0; i < 100; i++) {

;  	   v4[i] = (v1[i]+v2[i])/v3[i];

;}

| .data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Clock cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V1: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V2: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V3: .double “100 values” … V5: .double “100 zeros” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V4: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V5: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .text |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| main: daddui r1,r0,0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| daddui r2,r0,100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| loop: l.d f3,v1(r1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| l.d f2,v2(r1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| l.d f1,v3(r1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| add.d f4,f1,f2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| div.d f4,f4,f3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| s.d f4,v4(r1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| daddui r1,r1,8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| daddi r2,r2,-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| bnez r2,loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Halt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Total |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Domanda 2

Considerando il programma precedente, e in particolare la coppia di istruzioni:

add.d f4,f1,f2

div.d f4,f4,f3

che tipo di hazard crea l’utilizzo di f4 e come viene risolto? motivare la risposta.

Domanda 3

Considerando il programma precedente e l’architettura del processore superscalare descritto in seguito; completare la tabella relativa alle prime 3 iterazioni.

Processor architecture:

  - Issue 2 instructions per clock cycle

  - jump instructions require 1 issue

  - handle 2 instructions commit per clock cycle

  - timing facts for the following separate functional units:

    - 1 Memory address  1 clock cycle

    - 1 Integer ALU 1 clock cycle

    - 1 Jump unit 1 clock cycle

    - 1 FP multiplier unit, which is pipelined: 6 stages

    - 1 FP divider unit, which is not pipelined: 7 clock cycles

    - 1 FP Arithmetic unit, which is pipelined: 4 stages

  - Branch  prediction is always correct

  - There are no cache misses

  - There are 2 CDB (Common Data Bus).

| # iteration |  | Issue | EXE | MEM | CDB x2 | COMMIT x2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | l.d f3,v1(r1) |  |  |  |  |  |
| 1 | l.d f2,v2(r1) |  |  |  |  |  |
| 1 | l.d f1,v3(r1) |  |  |  |  |  |
| 1 | add.d f4,f1,f2 |  |  |  |  |  |
| 1 | div.d f4,f4,f3 |  |  |  |  |  |
| 1 | s.d f4,v4(r1) |  |  |  |  |  |
| 1 | daddui r1,r1,8 |  |  |  |  |  |
| 1 | daddi r2,r2,-1 |  |  |  |  |  |
| 1 | bnez r2,loop |  |  |  |  |  |
| 2 | l.d f3,v1(r1) |  |  |  |  |  |
| 2 | l.d f2,v2(r1) |  |  |  |  |  |
| 2 | l.d f1,v3(r1) |  |  |  |  |  |
| 2 | add.d f4,f1,f2 |  |  |  |  |  |
| 2 | div.d f4,f4,f3 |  |  |  |  |  |
| 2 | s.d f4,v4(r1) |  |  |  |  |  |
| 2 | daddui r1,r1,8 |  |  |  |  |  |
| 2 | daddi r2,r2,-1 |  |  |  |  |  |
| 2 | bnez r2,loop |  |  |  |  |  |
| 3 | l.d f3,v1(r1) |  |  |  |  |  |
| 3 | l.d f2,v2(r1) |  |  |  |  |  |
| 3 | l.d f1,v3(r1) |  |  |  |  |  |
| 3 | add.d f4,f1,f2 |  |  |  |  |  |
| 3 | div.d f4,f4,f3 |  |  |  |  |  |
| 3 | s.d f4,v4(r1) |  |  |  |  |  |
| 3 | daddui r1,r1,8 |  |  |  |  |  |
| 3 | daddi r2,r2,-1 |  |  |  |  |  |
| 3 | bnez r2,loop |  |  |  |  |  |

Domanda 4

Considerando il segmento di codice presentato nella tabella precedente, se assumiamo che ci sia un unico Common Data Bus, qual è la prima istruzione che dovrebbe stallare durante l’esecuzione del programma? motivare la risposta.