Domanda 1

Considerando il processore MIPS64 e l’architettura descritta in seguito:

| - Integer ALU: 1 clock cycle - Data memory: 1 clock cycle - FP multiplier unit: pipelined 6 stages | - FP arithmetic unit: pipelined 3 stages - FP divider unit: not pipelined unit that requires 6 clock cycles - branch delay slot: 1 clock cycle, and the branch delay slot disabled | - forwarding enabled - it is possible to complete instruction EXE stage in an out-of-order fashion. |
| --- | --- | --- |

Usando il frammento di codice riportato, si calcoli il tempo di esecuzione dell’intero programma in colpi di clock e si completi la seguente tabella.

;	for (i = 0; i < 100; i++) {

;		v4[i] = (v1[i] / v2[i]) + v3[i];

;		v5[i] = v4[i] * v3[i]; }

|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Clock cycles |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| .data |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V1: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V2: .double “100 values” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| V3: .double “100 values” … V5: .double “100 zeros” |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| .text |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| main: daddui r1,r0,0 | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| daddui r2,r0,100 |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| loop: l.d f1,v1(r1) |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| l.d f2,v2(r1) |  |  |  | F | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| div.d f4,f1,f2 |  |  |  |  | F | D | s | d | d | d | d | d | d | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |
| l.d f3,v3(r1) |  |  |  |  |  | F | s | D | E | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| dadd.d f4,f4,f3 |  |  |  |  |  |  |  | F | D | s | s | s | s | A | a | A | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |
| s.d f4,v4(r1) |  |  |  |  |  |  |  |  | F | s | s | s | s | D | E | S | S | M | W |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |
| mul.d f5,f4,f3 |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | s | X | x | x | x | x | x | M | W |  |  |  |  |  |  |  |  |  |  |  |  | 5 |
| s.d f5,v5(r1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s | D | E | s | s | s | s | s | M | W |  |  |  |  |  |  |  |  |  |  |  | 1 |
| daddui r1,r1,8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | D | s | s | s | s | s | E | M | W |  |  |  |  |  |  |  |  |  |  | 1 |
| daddi r2,r2,-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s |  |  |  |  | D | E | M | W |  |  |  |  |  |  |  |  |  | 1 |
| bnez r2,loop |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | s | D | E | M | W |  |  |  |  |  |  |  | 2 |
| Halt |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F | - | - | - | - | - | - |  |  |  |  | 1 |
| TOTAL = 6 + 100*(2+10+6+6) = |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2406 |

Domanda 2

Considerando il programma precedente, cosa succede se i salti vengono svolti durante la fase di EXE e non durante la fase di DECODE? Motivare la risposta.

Verrebbe fatta anche la DECODE Della Halt finale, e in generale si sprecherebbe un ciclo di clock aggiuntivo prima di saltare e caricare la l.d, per ogni salto.

Domanda 3

Considerando il programma precedente e l’architettura del processore superscalare descritto in seguito; completare la tabella relativa alle prime 3 iterazioni.

Processor architecture:

•	Issue 2 instructions per clock cycle

•	jump instructions require 1 issue

•	handle 2 instructions commit per clock cycle

•	timing facts for the following separate functional units:

i.	1 Memory address  1 clock cycle

ii.	1 Integer ALU 1 clock cycle

iii.	1 Jump unit 1 clock cycle

iv.	1 FP multiplier unit, which is pipelined: 6 stages

v.	1 FP divider unit, which is not pipelined: 6 clock cycles

vi.	1 FP Arithmetic unit, which is pipelined: 3 stages

•	Branch  prediction is always correct

•	There are no cache misses

•	There are 2 CDB (Common Data Bus).

| # iteration |  | Issue | EXE | MEM | CDB x2 | COMMIT x2 |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | l.d f1,v1(r1) | 1 | 2m | 3 | 4 | 5 |
| 1 | l.d f2,v2(r1) | 1 | 3m | 4 | 5 | 6 |
| 1 | div.d f4,f1,f2 | 2 | 6d | - | 12 | 13 |
| 1 | l.d f3,v3(r1) | 2 | 4m | 5 | 6 | 13 |
| 1 | dadd.d f4,f4,f3 | 3 | 13 a | - | 16 | 17 |
| 1 | s.d f4,v4(r1) | 3 | 5m | - | - | 17 |
| 1 | mul.d f5,f4,f3 | 4 | 17x | - | 23 | 24 |
| 1 | s.d f5,v5(r1) | 4 | 6m | - | - | 24 |
| 1 | daddui r1,r1,8 | 5 | 6i | - | 7 | 25 |
| 1 | daddi r2,r2,-1 | 5 | 7i | - | 8 | 25 |
| 1 | bnez r2,loop | 6 | 9j | - | - | 26 |
| 2 | l.d f1,v1(r1) | 7 | 8m | 9 | 10 | 26 |
| 2 | l.d f2,v2(r1) | 7 | 9m | 10 | 11 | 27 |
| 2 | div.d f4,f1,f2 | 8 | 12d | - | 18 | 27 |
| 2 | l.d f3,v3(r1) | 8 | 10m | 11 | 12 | 28 |
| 2 | dadd.d f4,f4,f3 | 9 | 19a | - | 22 | 28 |
| 2 | s.d f4,v4(r1) | 9 | 11m | - | - | 29 |
| 2 | mul.d f5,f4,f3 | 10 | 23x | - | 29 | 30 |
| 2 | s.d f5,v5(r1) | 10 | 12m | - | - | 30 |
| 2 | daddui r1,r1,8 | 11 | 12i | - | 13 | 31 |
| 2 | daddi r2,r2,-1 | 11 | 13i | - | 14 | 31 |
| 2 | bnez r2,loop | 12 | 15j | - | - | 32 |
| 3 | l.d f1,v1(r1) | 13 | 14m | 15 | 16 | 32 |
| 3 | l.d f2,v2(r1) | 13 | 15m | 16 | 17 | 33 |
| 3 | div.d f4,f1,f2 | 14 | 18d | - | 24 | 33 |
| 3 | l.d f3,v3(r1) | 14 | 16m | 17 | 18 | 34 |
| 3 | dadd.d f4,f4,f3 | 15 | 25 a | - | 28 | 34 |
| 3 | s.d f4,v4(r1) | 15 | 17m | - | - | 35 |
| 3 | mul.d f5,f4,f3 | 16 | 29x | - | 35 | 36 |
| 3 | s.d f5,v5(r1) | 16 | 18m | - | - | 36 |
| 3 | daddui r1,r1,8 | 17 | 18i | - | 19 | 37 |
| 3 | daddi r2,r2,-1 | 17 | 19i | - | 20 | 37 |
| 3 | bnez r2,loop | 18 | 21j | - | - | 38 |

Domanda 4

Considerando il segmento di codice presentato nella tabella precedente, se assumiamo che ci sia un unico Common Data Bus, qual è la prima istruzione che dovrebbe stallare durante l’esecuzione del programma? motivare la risposta.

Se è disponibile un solo CDB, la prima istruzione a stallare sarà

| 2 | l.d f3,v3(r1) | 8 | 10m | 11 | 12 | 28 |
| --- | --- | --- | --- | --- | --- | --- |

Questo perché il CDB sarà già occupato da un’istruzione che ancora non ha fatto commit e che sta già usando il CBD, ed è l’istruzione

| 1 | div.d f4,f1,f2 | 2 | 6d | - | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- |